# Component A: Rigid Body Geometry and Inertial Properties

This component defines the particle as a rigid body data structure that provides its surface mesh (for Part C force integration), center of mass, and inertia tensor (for Part D dynamics). Three levels of shape complexity are supported.

## A.1  Key Quantities

### Mass and Center of Mass

For uniform density ρ and volume V the mass is M = ρV. The center of mass in the body frame is:

r_cm  =  (1/M) ∫_V  r ρ(r) dV   (A.1)

For a mesh of N_f triangular faces, the volume and centroid are computed by the divergence theorem over face contributions.

### Inertia Tensor

The 3×3 inertia tensor I in the body frame (origin at r_cm) is:

I_ij  =  ∫_V  ρ ( |r|² δ_ij  −  r_i r_j )  dV   (A.2)

For composite bodies (aggregates) the parallel-axis theorem adds M_k(|d_k|²δ_ij − d_k,i d_k,j) per sub-body k displaced d_k from the composite center of mass. The principal moments of inertia are the eigenvalues of I; the principal axes are its eigenvectors and are used as the body-frame coordinate axes in Part D.

### Analytic Results for Primitive Shapes

| Item | Detail |
| --- | --- |
| Sphere (radius R) | M=(4/3)πR³ρ, I=(2/5)MR² · I₃ |
| Prolate spheroid (a,a,c) | I_xx=I_yy=M(a²+c²)/5, I_zz=2Ma²/5 |
| Cylinder (R, L) | I_xx=I_yy=M(3R²+L²)/12, I_zz=MR²/2 |
| Sphero-cylinder (R, L) | Sum of cylinder shaft + two hemispherical caps via parallel axis |
| Two-sphere aggregate | Parallel axis theorem applied to each sphere |

## A.2  Surface Mesh

Part C integrates the thermophoretic stress over the particle surface. The surface is stored as arrays of face centroids r_f, outward unit normals n_f, and areas dA_f. The surface integral ∫ g dA is approximated as:

∫_Σ g(r, n) dA   ≈   Σ_f  g(r_f, n_f) · dA_f   (A.3)

### Shape Levels

- Level 1 — Analytic primitives (sphere, spheroid, sphero-cylinder): closed-form surfaces, meshed with trimesh.creation.icosphere / cylinder + caps.

- Level 2 — Implicit surface f(x,y,z)=0: extract with skimage.measure.marching_cubes on a 3-D grid.

- Level 3 — External STL/OBJ file: load with trimesh.load(), then trimesh.repair.fix_normals() for consistent outward normals.

## A.3  Python Implementation

### Data Class

import numpy as np

import trimesh

from dataclasses import dataclass

@dataclass

class RigidBody:

mesh:          trimesh.Trimesh   # surface mesh in body frame

rho:           float             # density [kg/m³]

mass:          float             # M [kg]

r_cm:          np.ndarray        # (3,) center of mass, body frame [m]

I_body:        np.ndarray        # (3,3) inertia tensor, body frame [kg·m²]

# --- derived surface arrays (set once at init) ---

face_centroids: np.ndarray       # (N_f, 3) [m]

face_normals:   np.ndarray       # (N_f, 3) outward unit normals

face_areas:     np.ndarray       # (N_f,)   [m²]

### Factory Functions

def _build(mesh, rho):

"""Compute mass, r_cm, I from a trimesh object."""

vol    = mesh.volume

mass   = rho * vol

r_cm   = mesh.center_mass           # trimesh computes this

# Shift mesh origin to r_cm for inertia calculation

I_body = trimesh.inertia.mesh_inertia(mesh) * rho

return RigidBody(

mesh=mesh, rho=rho, mass=mass, r_cm=r_cm, I_body=I_body,

face_centroids = mesh.triangles_center,

face_normals   = mesh.face_normals,

face_areas     = mesh.area_faces,

)

def make_sphere(R, rho):

return _build(trimesh.creation.icosphere(subdivisions=4, radius=R), rho)

def make_spherocylinder(R, L, rho):

cyl      = trimesh.creation.cylinder(radius=R, height=L)

cap_top  = trimesh.creation.hemisphere(radius=R)

cap_bot  = trimesh.creation.hemisphere(radius=R)

# translate caps to ±L/2 along z, flip bottom cap

cap_top.apply_translation([0, 0,  L/2])

cap_bot.apply_transform(trimesh.transformations.rotation_matrix(np.pi, [1,0,0]))

cap_bot.apply_translation([0, 0, -L/2])

mesh = trimesh.util.concatenate([cyl, cap_top, cap_bot])

return _build(mesh, rho)

def make_from_stl(path, rho):

mesh = trimesh.load(path)

trimesh.repair.fix_normals(mesh)

return _build(mesh, rho)

## A.4  Benchmarks

| Item | Detail |
| --- | --- |
| Sphere inertia | I = (2/5)MR²·I₃ (analytic vs mesh < 0.5 %) |
| Prolate spheroid I_zz | 2Ma²/5 (analytic vs mesh < 0.5 %) |
| Two-sphere aggregate r_cm | On line of centers, weighted by mass |
| Outward normals | n·(r−r_cm) > 0 for all faces |
| Mesh volume vs analytic | Agreement < 1 % |