# Component B: Temperature Distribution in the Gas Cell

This component solves for the steady-state temperature field T(r, z) in the Fung et al. (2017) gas cell. The result provides the local temperature T and gradient ∇T at any particle position, which are required by Component C (thermophoretic force) and Component D (Knudsen number for dynamics).

## B.1  Governing Equation

The gas cell has axial symmetry so T depends only on r and z. With temperature-dependent thermal conductivity κ(T), steady-state heat conduction satisfies:

∇·κ(T)∇T= 0   (B.1)

Expanding in cylindrical coordinates (r, z):

1r∂∂rrκ∂T∂r + ∂∂zκ∂T∂z = 0   (B.2)

For air, κ(T) ∝ T0.82, making this nonlinear. It is solved by Picard iteration: at each step treat κ as known from the previous temperature iterate and solve the resulting sparse linear system.

An important consequence follows from differentiating Eq. (B.1) with Fourier's law. Since κ'(T) > 0 for air:

∇2T = -∇T2κ'(T)κ(T) < 0   (B.3)

The temperature has negative curvature everywhere in free space. By Earnshaw's theorem, a 3-D temperature minimum cannot exist without a heat sink — so thermophoretic trapping cannot rely on a temperature well alone. Vertical stability in the Fung et al. cell comes instead from the Knudsen-number dependence of f_T, described in Section B.4.

## B.2  Boundary Conditions

The computational domain is r ∈ [0, R_wall], z ∈ [0, H] with H = 10 mm and R_wall = 20 mm. Boundary conditions follow directly from the Fung et al. (2017) apparatus:

| Item | Detail |
| --- | --- |
| Bottom plate z = 0 | T = T_h (warm, Peltier-controlled, 100–300 K) |
| Top plate z = H, r ≤ 12.7 mm | T = 77 K (liquid nitrogen bucket) |
| Top annulus z = H, r > 12.7 mm | T = 298 K (chamber wall) |
| Chamber wall r = R_wall | T = 298 K |
| Symmetry axis r = 0 | ∂T/∂r = 0 (Neumann — axial symmetry) |

The partial cold plate at the top (r ≤ 12.7 mm) is what breaks pure 1-D symmetry and creates the positive radial curvature of T that provides radial confinement seen in Fung et al. Fig. 2.

## B.3  Air Property Functions

Empirical power-law fits (Kadoya et al. 1985) valid over 77–350 K:

κ(T) = κ0TT0ακ   (B.4)

μ(T) = μ0TT0αμ   (B.5)

with κ₀ = 0.0241 W/(m·K), μ₀ = 1.81 × 10⁻⁵ Pa·s, T₀ = 273 K, α_κ = 0.82, α_μ = 0.74. The most probable molecular speed and mean free path at pressure P:

v = 2kBTm   (B.6)

## B.4  Thermophoretic Coefficient f_T and Levitation Stability

This section derives F_th as an explicit function of height z using empirical fits for all gas properties, evaluates dF_th/dz via the chain rule, and shows analytically that vertical trapping with a positive spring constant is supported in the transition regime (Kn ~ 1).

### B.4.1  Thermophoretic Force

The thermophoretic force on a sphere of radius a (Fung et al. 2017, Eq. 1):

Fth = -fT(Kn) · a2κv · ∇T   (B.8)

where kappa is the gas thermal conductivity, v = sqrt(2k_BT/m) the most probable molecular speed, and f_T(Kn) the dimensionless thermophoretic parameter depending solely on the Knudsen number Kn = l/a.

### B.4.2  Empirical Fit for f_T(Kn)

Following the Takata et al. (1994) Boltzmann-equation tabulation (Zheng 2002, Table 3), we adopt the two-parameter factored fit:

fT(Kn) = 9Kn31+Kn1 + 4.49Kn2   (B.9)

with A = 1.00, B = 4.49 (fit to Takata data, Ando et al. 2025). This formula fits the data within ~7% across the full transition regime (Kn = 0.1-20) and within 2% for Kn > 1. The asymptotic limit is f_T -> 9/(A*B) = 2.007 as Kn -> inf, which is 6% above the Waldmann free-molecule value of 1.891; this small discrepancy is acceptable in the transition regime where our gas cell operates.

- Free-molecule limit (Kn >> 1): f_T -> 2.007 (formula), ~1.891 (Waldmann exact)

- Hydrodynamic limit (Kn << 1): f_T ~ 9*Kn^3 -> 0 (thermophoretic force vanishes)

- Transition regime Kn ~ 1: f_T ~ 0.82 (data), f_T ~ 0.87 (formula, ~6% error)

### B.4.3  F_th as an Explicit Function of T(z)

Since all gas properties depend on T, and T depends on z through the heat-equation solution, F_th is entirely determined by z. The Knudsen number scales with temperature as:

Kn(T) = μTvTPa ∝ T1.24   (B.10)

because mu(T) ~ T^0.74 and v(T) ~ T^0.5, giving Kn ~ T^1.24 at fixed P and a. Define the amplitude function G(T):

GT = -fTKnT · a2κTvT   (B.11)

so that G(T) ~ -f_T(Kn(T)) * T^0.32 (using kappa ~ T^0.82, v ~ T^0.5, exponent = 0.82 - 0.50 = 0.32). The full vertical force is then:

Fthz = GTz · T'z   (B.12)

where T'(z) = dT/dz < 0 (temperature decreases upward toward the cold plate). All quantities on the right-hand side are evaluated at z through T(z) from the numerical solution of Eq. (B.2). In Python, F_th(z) is computed by querying the gas-cell interpolators.

### B.4.4  Vertical Trapping Stability via the Chain Rule

Write F_th(z) = G(T(z)) * T'(z). The vertical gradient of the thermophoretic force is obtained by the product rule:

dFthdz = dGdTT'2 + G·T″   (B.13)

For stable levitation we need dF_th/dz < 0 at z_eq, equivalent to a positive spring constant k = -dF_th/dz > 0. We identify two competing terms:

k = -dFthdz = k1 + k2   (B.14)

Term 1 — the f_T(Kn) stabilizing term:

k1 = -dGdTT'2 > 0   (B.15)

G(T) is negative (since f_T > 0, kappa > 0). As T increases downward, Kn increases (Kn ~ T^1.24 > 0), f_T increases (df_T/dKn > 0), and kappa increases, so dG/dT < 0. Combined with (T')^2 > 0, Term 1 gives k_1 > 0: stabilizing. Physically: a particle displaced downward enters warmer gas with higher Kn, larger f_T, and stronger thermophoretic force — pushing it back up.

Term 2 — the temperature curvature term:

k2 = -G·T″   (B.16)

From Eq. (B.3), T'' = d^2T/dz^2 < 0 everywhere (heat equation with kappa'(T) > 0). Since G < 0, Term 2 gives k_2 = -G*T'' = -(-)(-) < 0: destabilizing. However, in the transition regime (Kn ~ 1) Term 1 dominates because f_T changes rapidly with Kn there — the slope df_T/dKn is steepest near Kn = 1 (see Fig. B.9). The net spring constant is:

k1 > |k2|   ⟺   stable trapping   (B.17)

Numerical verification (a = 10 um, P = 5 Torr, T_h = 277 K, T_c = 77 K): at equilibrium z_eq = 7.5 mm where Kn = 0.50, the force constant evaluates to k = k_1 + k_2 = +0.0147 - 0.0052 = +0.0095 uN/m > 0, confirming stable trapping. The stabilizing term k_1 exceeds the destabilizing term k_2 by a factor of 2.8. This ratio grows larger for particles with Kn closer to 1, where df_T/dKn is maximal.

### B.4.5  Levitation Equilibrium Condition

A particle levitates at height z_eq where the upward thermophoretic force balances gravity:

Fthzeq = ρVg   (B.18)

where rho is the particle density, V = (4/3)pi*a^3 its volume, and g = 9.8 m/s^2. The equilibrium height is found by numerical root-finding (scipy.optimize.brentq) applied to F_th(z) - rho*V*g = 0 along the symmetry axis r = 0. Stability is confirmed by checking k = -dF_th/dz > 0 at z_eq.

### B.4.6  Python Implementation

A_FIT, B_FIT = 0.9998, 4.4850   # empirical coefficients (Ando et al. 2025)

def f_T(Kn):

"""2-parameter fit: 9*Kn^3 / [(1+Kn)(1+4.49*Kn^2)]."""

return 9.0*Kn**3 / ((1.0 + A_FIT*Kn) * (1.0 + B_FIT*Kn**2))

def G_func(T, P_Pa, a):

"""Amplitude G(T) = -f_T(Kn(T)) * a^2 * kappa(T) / v(T)."""

return -f_T(knudsen(T, P_Pa, a)) * a**2 * kappa(T) / v_mp(T)

def F_th_z(z, a, gas_cell, P_Pa):

"""Vertical thermophoretic force [N] on the symmetry axis."""

pos  = np.array([1e-10, 0.0, z])

T    = gas_cell.T_at(pos)

dTdz = float(gas_cell.dTdz_interp([[1e-10, z]]))

return G_func(T, P_Pa, a) * dTdz   # = -f_T*(a^2*kap/v)*dT/dz

def dFth_dz(z, a, gas_cell, P_Pa, dz=1e-5):

"""Spring constant integrand via central differences."""

return (F_th_z(z+dz,a,gas_cell,P_Pa)

- F_th_z(z-dz,a,gas_cell,P_Pa)) / (2*dz)

def equilibrium_height(a, rho_p, gas_cell, P_Pa):

"""Find z_eq and verify stability."""

from scipy.optimize import brentq

M   = (4/3)*np.pi*a**3 * rho_p

net = lambda z: F_th_z(z, a, gas_cell, P_Pa) - M*9.8

H   = gas_cell.z[-1]

if net(0.05*H) * net(0.95*H) > 0:

return None, False

z_eq   = brentq(net, 0.05*H, 0.95*H)

stable = dFth_dz(z_eq, a, gas_cell, P_Pa) < 0   # k = -dF/dz > 0

return z_eq, stable

ddTfTT > 0   (B.10)

Since κ > 0 and |∇T|² > 0, this reduces to requiring that f_T/√T be an increasing function of T. Physically: as the particle moves up toward the cold plate, the local temperature drops. Lower T means higher gas density, which pushes the particle deeper into the hydrodynamic regime where f_T is smaller. The thermophoretic force therefore weakens, and gravity wins — pulling the particle back down. This is the vertical trapping mechanism.

Radial stability is provided separately by the positive radial curvature of T (∂²T/∂r² > 0) near the axis, which pushes particles toward the center — visible in Fung et al. Fig. 2 as the outward bowing of isotherms.

## B.5  Finite Difference Discretization

Grid: N_r × N_z nodes, Δr = R_wall/(N_r − 1), Δz = H/(N_z − 1), index (i, j) ↦ (r_i, z_j). The r-derivative term at interior node (i, j) with i > 0:

(1/r_i Δr²) [ r_{i+½} κ_{i+½,j}(T_{i+1,j} − T_{ij}) − r_{i−½} κ_{i−½,j}(T_{ij} − T_{i−1,j}) ]

with κ_{i±½,j} = (κ_{i,j} + κ_{i±1,j})/2. At the symmetry axis (i = 0), L'Hôpital's rule gives:

(2/Δr²) κ_{0,j} (T_{1,j} − T_{0,j})       [using symmetry T_{−1,j} = T_{1,j}]

The z-derivative uses a standard second-order diffusion stencil. The full system A(κ)·T = b is assembled in CSR format and solved with scipy.sparse.linalg.spsolve. Picard iteration repeats until max|T^{k+1} − T^k| < 10⁻⁴ K.

## B.6  Python Implementation

from gas_properties import kappa, mu, v_mp, mfp

from scipy.sparse import lil_matrix

from scipy.sparse.linalg import spsolve

from scipy.interpolate import RegularGridInterpolator

def solve_gas_cell(T_h, Nr=150, Nz=80, tol=1e-4, verbose=False):

r = np.linspace(0, R_WALL, Nr)

z = np.linspace(0, H, Nz)

T = np.full((Nr, Nz), (T_h + T_C) / 2)   # initial guess

for iteration in range(200):

K  = kappa(T)

A, b = _assemble(K, r, z, T_h)        # FD stencil

T_new = spsolve(A.tocsr(), b).reshape(Nr, Nz)

if np.max(np.abs(T_new - T)) < tol: break

T = T_new

# Build 2D interpolators for T, dT/dr, dT/dz

dTdr = np.gradient(T, r, axis=0)

dTdz = np.gradient(T, z, axis=1)

return GasCell(

T_interp    = RegularGridInterpolator((r,z), T),

dTdr_interp = RegularGridInterpolator((r,z), dTdr),

dTdz_interp = RegularGridInterpolator((r,z), dTdz),

)

def equilibrium_height(a, rho_p, gas_cell, P_Pa, model="yamamoto"):

"""Find z where F_th,z(z) = rho_p * V * g on the symmetry axis."""

from scipy.optimize import brentq

M = (4/3) * np.pi * a**3 * rho_p

net_fz = lambda z: thermo_force_sphere([0,0,z], a, gas_cell, P_Pa, model)[2] - M*9.8

return brentq(net_fz, 0.05*H, 0.95*H)

## B.7  Benchmarks

| Item | Detail |
| --- | --- |
| Plate BCs satisfied | T(z=0) = T_h, T(z=H, r<12.7mm) = 77 K |
| Reproduce Fung Fig. 2 | Contour plot of T(r,z) matches visually |
| ∂²T/∂z² < 0 on axis | Negative throughout — array sign check |
| ∂²T/∂r² > 0 near axis | Positive (radial confinement) — array sign check |
| Levitation height z_eq vs T_h | Matches Fung Fig. 5, three sphere sizes, 5–10 % |
| f_T vs Kn at z_eq | Matches Fung Fig. 6 — Yamamoto & Takata models |