# Component D: Six-Degree-of-Freedom Dynamical Evolution

Given forces and torques from Part C this component integrates the full Newton-Euler equations for the particle's translational position r_cm(t) and orientation q(t). The state vector has 13 components; an adaptive RK45 integrator advances it in time.

## D.1  Orientation: Unit Quaternions

Orientation is stored as a unit quaternion q = (q₀, q₁, q₂, q₃) with |q|=1. The rotation matrix from body to lab frame is:

R_ij(q)  =  δ_ij(q₀²−|q̄|²)  +  2q_i q_j  +  2q₀ ε_ijk q_k

where q̄ = (q₁,q₂,q₃). The quaternion evolves as:

q̇  =  ½  q ⊗ [0, ω_body]   (D.1)

where ω_body is the angular velocity in the body frame and ⊗ is quaternion multiplication. After each integrator step, q is renormalized: q ← q/|q|. Using quaternions avoids gimbal lock and is computationally cheaper than rotation matrices for integration.

## D.2  Translational Equation of Motion

Newton's second law in the lab frame:

M r̈_cm  =  F_th(r_cm, q)  +  F_gravity  +  F_drag(ṙ_cm, q)   (D.2)

The three forces are:

- F_th: thermophoretic force (Part C), depends on position and orientation

- F_gravity = (0, 0, −Mg),   g = 9.8 m/s²

- F_drag = −R^T ξ_t R ṙ_cm  (anisotropic Stokes drag, C_c slip correction)

The inertial relaxation time τ_t = M/ξ_t ~ ρ_p a²/μ is ~10⁻⁴ s for a 10 μm sphere — much shorter than the levitation dynamics (~seconds). The particle is therefore overdamped, and r̈_cm ≈ 0 is a valid simplification for steady-state calculations. The full second-order form is retained for generality.

## D.3  Rotational Equation of Motion (Euler's Equations)

In the body frame, with I = diag(I₁, I₂, I₃) the principal inertia tensor:

I ω̇_body  +  ω_body × (I ω_body)  =  τ_body   (D.3)

where τ_body = R^T(τ_th + τ_drag) is the total torque in the body frame and τ_drag = −ξ_r ω_body. Euler's equations expanded per axis:

I₁ ω̇₁  =  τ₁  −  (I₃−I₂) ω₂ω₃

I₂ ω̇₂  =  τ₂  −  (I₁−I₃) ω₃ω₁

I₃ ω̇₃  =  τ₃  −  (I₂−I₁) ω₁ω₂

## D.4  State Vector and ODE System

Collect all degrees of freedom into a 13-component state vector:

s  =  [ r_cm (3),   ṙ_cm (3),   q (4),   ω_body (3) ]

The derivative function deriv(t, s) computes ṡ = [ṙ_cm, r̈_cm, q̇, ω̇_body] by:

- Unpack and renormalize q.

- Compute R = R(q).

- Query Part B: T, ∇T at r_cm.

- Call Part C: F_th (lab) and τ_th (lab) for current orientation.

- Compute F_drag = −R^T ξ_t R ṙ_cm  and  τ_drag = −R^T ξ_r R ω_lab.

- r̈_cm = (F_th + F_gravity + F_drag) / M.

- τ_body = R^T(τ_th) + τ_drag,body  →  solve Euler Eq. D.3 for ω̇.

- q̇ = ½ q ⊗ [0, ω_body].

## D.5  Python Implementation

### Quaternion utilities

import numpy as np

def quat_mult(q1, q2):

w1,x1,y1,z1 = q1;  w2,x2,y2,z2 = q2

return np.array([w1*w2-x1*x2-y1*y2-z1*z2,

w1*x2+x1*w2+y1*z2-z1*y2,

w1*y2-x1*z2+y1*w2+z1*x2,

w1*z2+x1*y2-y1*x2+z1*w2])

def quat_to_matrix(q):

w,x,y,z = q / np.linalg.norm(q)

return np.array([[1-2*(y*y+z*z), 2*(x*y-w*z),   2*(x*z+w*y)],

[2*(x*y+w*z),   1-2*(x*x+z*z), 2*(y*z-w*x)],

[2*(x*z-w*y),   2*(y*z+w*x),   1-2*(x*x+y*y)]])

### ODE right-hand side

from scipy.integrate import solve_ivp

def deriv(t, s, body, gas_cell, P, model):

r_cm = s[0:3];  v_cm = s[3:6]

q    = s[6:10] / np.linalg.norm(s[6:10])   # renormalize

omg  = s[10:13]                             # body frame

Rmat = quat_to_matrix(q)

# Forces and torques from Part C

F_th, tau_th = thermo_force_mesh(body, r_cm, q, gas_cell, P,

a_eq=body.r_eq, model=model)

# Drag

Kn   = mfp(gas_cell.T_at(r_cm), P) / body.r_eq

Cc   = cunningham(Kn)

xi_t = 6*np.pi * mu(gas_cell.T_at(r_cm)) * body.r_eq / Cc

xi_r = 8*np.pi * mu(gas_cell.T_at(r_cm)) * body.r_eq**3 / Cc

F_drag = -xi_t * v_cm                       # sphere approx; extend for mesh

F_grav = np.array([0.0, 0.0, -body.mass * 9.8])

a_cm   = (F_th + F_drag + F_grav) / body.mass

# Euler equations in body frame

tau_body = Rmat.T @ tau_th - xi_r * omg

I = np.diag(np.linalg.eigvalsh(body.I_body))  # principal moments

omg_dot  = np.linalg.solve(

np.diag(I),

tau_body - np.cross(omg, I * omg))

q_dot = 0.5 * quat_mult(q, np.r_[0.0, omg])

return np.r_[v_cm, a_cm, q_dot, omg_dot]

def simulate(body, gas_cell, P, s0, t_span, model="yamamoto"):

return solve_ivp(

deriv, t_span, s0, method="RK45",

args=(body, gas_cell, P, model),

rtol=1e-6, atol=1e-9, dense_output=True)

### Equilibrium height (fast path, sphere only)

from scipy.optimize import brentq

def equilibrium_height(a, rho_p, gas_cell, P, T_h, model="yamamoto"):

"""Find z_eq where F_th,z = Mg. Searches on axis (r=0)."""

M = (4/3)*np.pi*a**3 * rho_p

def net_fz(z):

pos  = np.array([1e-9, 0, z])   # tiny r to avoid 0/0

F_th = thermo_force_sphere(pos, a, gas_cell, P, model)

return F_th[2] - M*9.8

# Search between z=0.5mm and z=H-0.5mm

H = 0.01

if net_fz(H*0.05) * net_fz(H*0.95) > 0:

return None    # no crossing

return brentq(net_fz, H*0.05, H*0.95)

## D.6  Benchmarks

| Item | Detail |
| --- | --- |
| Sphere z_eq vs T_h ramp | Matches Fung Fig. 5, 3 sphere sizes, 5–10 % |
| Terminal velocity (sphere) | v_T = F_th / (6πμa/C_c) |
| No rotation in uniform ∇T | ¦ω¦ < 10⁻¹⁰ rad/s (symmetric body) |
| ¦q¦ = 1 maintained | Drift < 10⁻¹⁰ over 10⁴ steps |
| Sphero-cyl parallel to ∇T | Drifts ~31% faster than equiv. sphere (G-Y&R) |
| Drift angle at oblique β | Matches Garcia-Ybarra & Rosner Fig. 4 |
| Asymmetric aggregate rotation | Larger sphere rotates to upstream (hot) side |